- SCope of HOIST AND CRANE INSPECTION, REPAIR, SERVICES AND TRAINING.

  - General Scope.

The Charlotte Area Transit System (CATS) must maintain its hoist and crane equipment to service their facilities and ongoing projects. The hoist and crane equipment must be inspected for safety, full-service maintenance provided, and repairs made.

The general scope of the offered three-year (3 years) contract shall be to perform basic OSHA inspections, safety inspections, full-service preventive maintenance, repairs of its hoist and crane equipment, and training.

While the City is flexible with respect to certain elements of the HOIST AND CRANE REPAIR, SERVICES AND TRAINING, the City has specific requirements and preferences for the Service delivery method.

  - Inspections and Reporting.

    - Each inspection shall comply with OSHA 1910.179.

    - Each inspection shall comply with all applicable regulations regarding the inspection of hoist and cranes.

    - OSHA inspections to include, but not limited to, the following:

1.	Inspections will be conducted by a factory-trained technician who        specializes in crane systems, service, and inspections;

2.        Electrical and mechanical inspections of hoist, trolley, crane, runway, and runway structural supports;

3.         Wearing parts such as wire ropes, chain, wheels, contactors, and collectors;

4.	Hoist, trolley, and bridge motors will be inspected for correct oil levels;

5.	Sheaves, drums, wheels, bearings for general condition and lubrication. Wire ropes and chains will be lubricated if use of the lubricant is approved by the customer;

6.	Visually inspect load block and hook for excessive wear such as bending, twisting, cracks, grooves, or Increased throat opening;

7.	External parts of the hoist, trolley, frame, catwalks, and handrails for broken parts, mis-alignments, broken welds, excessive wear, or any unusual conditions;

8.	Control boxes and check all contactors, relays, timers, loose/broken connections, and pitting for proper operations;

9.	Safety devices, limit switches, warning lights, and horns for proper operation;

10.	All supporting steel and runways for general condition, broken welds, loose connections, end stops, and "J" bolts etc.;

11.	External motors, couplings and wiring for wear and deterioration and externally check the motors for proper wear and operation; and

12.	Inspection of the motor brakes for the following:

a.  Excessive drift;

b.  Oil  leaking Into brake components;

c.  Excessive wear of pads;

d.  Damaged components;

e.  Burnt coils;

f.  Air gap adjustment incorrect components;

g.  Brake shoes on drum not in alignment; and

h.  Brake drum scored or damaged.

    - Pricing for the inspections, parts, and repairs shall be included in the Pricing Sheet found in the Required Forms in this RFP.

    - Contractor is to perform inspections every three (3) months on all equipment as directed by each department representative.  Each bidder shall submit a detailed description of the items covered on their proposed inspection found in the Required Forms attached to this RFQ.

  - Reporting Requirements.

    - Recommended repairs resulting from the inspections, if any, shall be indicated during the inspection visit and documented in the report.

    - A detailed report shall be prepared based on the results of the inspections described above. All Required Forms must be included for the bids to be considered..

    - Recommended repairs resulting from the inspections, if any, shall be indicated during the inspection visit and documented in the report.

  - Bidding and Pricing Requirements.

- Bid prices for inspections shall be a fixed price as indicated on the bid proposal Pricing Sheets.

b.	Bid prices for maintenance and repair shall be based on labor and material pricing as described in these specifications.

c.	A detailed repair cost shall be submitted to the department representative based on the terms of the contract.

d.	All repair work must be authorized before work is started.

e.	Each bidder must indicate an all-inclusive, flat-rate price for the inspection of each crane listed in these specifications.

f.	No separate payment shall be considered for labor, tools, small equipment, bucket trucks, man-lifts, or any other costs associated with inspection of the cranes.

g.	Failure to submit a bid on any single line item may result in rejection of the bid in its entirety.

h.	Each bidder must indicate total hourly wage rates on Pricing Sheet that must be included with a bid to be considered.

i.	Wage rates shall include all wage classifications involved and shall include all tools and small equipment needed to perform necessary work.

j.	Separate payment shall be made for a bucket truck to allow personnel access to complete repairs and maintenance per the bid line for this item. Proposal must include minimum bucket truck hours and travel time costs and hourly bucket truck rate.

- There shall be no mark-up allowed on rented equipment.

m.	Pricing for repair parts and lubricants shall be at the vendor's acquisition cost plus the mark­ up percentage as indicated in the line item bid.

n.	Original invoices from the suppliers of the parts or lubricants used in repairs and maintenance shall be submitted to the City with any application for payment for repair work.

o.	No payment for parts or lubricants used in repairs and maintenance shall be made without these invoices.

p.	Pricing for load tests shall be based on the labor and material provisions contained in the specifications.

q.	The City shall make no separate payment for vendor travel time or associated vendor transportation costs.

r.	The City shall only pay for vendor time spent on a site actively inspecting or performing repairs to a crane or hoist. There shall be no exceptions.

s.	Bidders must provide pricing according to the Pricing Sheets and Required forms in this RFP.

  - Response Time.

Response time on call back repairs shall be three (3) hours at a maximum.

Response time is defined as having sufficient resources  on the job site to perform

required repairs.

  - Training Plan.

    - Onsite hoist and/or crane operator training

    - Two to three hours' sessions (dependent upon the number of attendees)

    - OSHA daily overhead hoist and/or crane operator training

    - OSHA safety requirements for operation of overhead hoists and/or crane

    - CMAA  proper operation of overhead hoists and/or crane

    - Presentation, training materials, and certifications (e.g. PowerPoint Presentations, Training Booklets, Test Papers, Certificates (of completion or passing), Operator Cards

  - Minimum Qualifications.

The Contractor/ Vendor must be certified by a nationally recognized organization, OSHA (Occupational Safety and Health Administration), and AMSE (American Society of Mechanical Engineers).  Contractor/Vendor must also be trained to inspect according to the applicable regulations of OSHA 1910.179 and ASME B30.2-2011. Contractor/Vendor should have at least five (5) years training in basic OSHA inspections, safety inspections, full service preventive maintenance, and repairs.

  - Warranty.

All parts, materials, and labor used on repair jobs must be new, of latest manufacture and guaranteed against defects in material and workmanship for a minimum of one (1) year. The bidder must state the warranty offered.

  - Work on City Premises.

Whenever the Contractor/Vendor, including personnel, agents, and subcontractors, is on the City's premises, he/she will obey all instructions and directions issued by the City's Project Manager and/or designee. The Contractor/Vendor also agrees they will comply with all rules, regulations, and security procedures of the City. The Contractor/Vendor shall maintain a clean and safe job site and restore all disturbed areas to their pre-existing condition.